Name_______________________ Student ID ____________________ Date: __________

Duration: 1h 15min, Format: Closed Book

Resources:

- Labtest2_practice_template.zip

- Lab sheets 1/2/3/4/5 (Hard copies provided by lab)

Instructions:

- Extract Labtest2_practice_template.zip to D:\PXXXXXXX_Your_name

- You are expected to interface to the Conveyor Kit when answering questions. Strictly refer to the Tag name & hardware address below.

- Complete the program and save it. Demonstrate to Instructor.

- Students shall not use Network and Mobile devices during the test.

- Students shall not remove any papers from the test venue.

Address for Conveyor System

Sensor_Start
%I127.1

Sensor_End
%I127.2

Sensor_Inductive
%I127.3

Solenoid
%Q129.2

Sensor_Slide
%I127.0

Motor_FWD %Q129.0
Motor_REV %Q129.1

Sensor_Start
%I127.1

Sensor_End
%I127.2

Sensor_Inductive
%I127.3

Solenoid
%Q129.2

Sensor_Slide
%I127.0

Motor_FWD %Q129.0
Motor_REV %Q129.1

Create the PLC program and link it to the HMI program provided. You are required to add the missing operation_lamp to the HMI.

Create MISSING
Operation
Lamp

Create MISSING
Operation
Lamp

| s/n | Requirements |  | Marks |
| --- | --- | --- | --- |
| 1 | Placing workpiece at sensor_start should turn its indicator ON(green) (this proves the sensor and HMI works) Removing the workpiece turns it OFF(grey) *Hint: refer to Lab5 Task 2 to learn how to link indicators and button on screen to PLC program. *Hint: if you are practicing using simulation instead of lab equipment, sensor_start can be change to %M instead of %I, so you can modify it easily. | 10 |  |
| 2 | Before operation begins, the operation_lamp is red (operation lamp is missing from your HMI, you have to create it) | 10 |  |
| 3 | When workpiece is placed at sensor_start & start_button is pressed, operation_lamp turns green and stays green throughout the process. | 10 |  |
| 4 | Forward / Reverse arrow indicator turns green whenever the conveyor moves in the respective direction. Otherwise, they are grey. | 5 |  |
| 5 | The sequence is as follows: - forward for 7 seconds - stops for 10 second for painting to take place, Spray_paint should be ON(blue) during this time. - reverse until workpiece reaches sensor_start. *Hint: use timer, refer to Lab2. | 20 |  |

| 6 | At this instance, the operation_lamp will turn back to red. | 5 |  |
| --- | --- | --- | --- |
| 7 | At the same time, Remove_painted_part indicator will blink green at 0.5Hz (i.e 1sec on, 1sec off). *Hint: use blinking bits. Refer to Lab3: Task1 | 10 |  |
| 8 | When a worker collects the painted part by pressing Acknowledge button, the indicator above will stop blinking. | 5 |  |
| 9 | The above process can be repeated without problems. (Marks will be deducted if the process can repeated before the Acknowledge button is pressed in item(7)) | 10 |  |
| 10 | The number_of_painted_parts should be displayed correctly on screen. *Hint: use counter. Refer to Lab3. *Hint: to display counter on screen, Refer to Lab5: Task4 | 10 |  |
| 11 | The number_of_painted_parts can be reset at any time using the clear button. | 5 |  |